To Make Biologically Active Compost:

Setup:

- A place with easy access to water, perhaps in the shade of a tree, plenty of space to work the pile from one spot to another (as you are turning)

- 2 wooden pallets with hardware cloth stapled onto them. This allows air circulation from below the pile and the hardware cloth prevents the materials from falling through. 2 pallets allow you to move back and forth between them as you turn

- One canvas tarp to put over pile to keep out rain (canvas breathes and plastic doesn’t): heavy duty Cartman tarp 6’x8’ from Amazon

- One plastic tarp to put on the ground for use as you are turning your materials, to keep them uncontaminated during turning

- 1-3 compost thermometers, at least 24”—Reotemp makes good ones

- Geobin or hardware cloth ¼ or ½” mesh 3’x11’ to hold compost pile in place

- Bungee cords to hold hardware cloth together

- 40 or more 5-gallon buckets or equivalent to hold materials plus water—used to measure your categories of materials and also to soak in water prior to putting pile together

- Manure/pitch fork

Materials:

Rules: diversity of materials is key, the more kinds of materials the better. Typically and depending on ambient temps, 60% “browns,” 30% “greens,” and 10% high nitrogen. We might go higher on high nitrogen depending on night temps forecast.

- Browns: wood chips, broken down leaves, straw, shredded cardboard, stems of plants, etc.

- Greens: any plants cut green (dried or not), food scraps, etc.

- High N: cracked grains, alfalfa or other high protein hay, some manures (be careful of de-wormers, etc), fresh beer mash. Tractor Supply Standlee alfalfa works well but have one or more additional sources also.

Process:

The day before microbe pile making day: put all materials into 40-plus buckets and soak. Set aside time daily for checking pile temps at least twice/day for at least 2 weeks. Plan on 2 hours at least twice during first 7-10 days to turn pile the minimum number of times. Pile will sit until it cools and fungi grow—time for this dependent on materials in pile, etc.