To Make Biologically Active Compost Part II:

More on Process

Getting ready—setup and soaking your materials:

- When your site is ready, put out your 40 or more buckets as close to the geobin or wire  setup and plastic tarp as possible—so you don’t have to carry the full buckets far to dump them onto your tarp when it’s time to build your pile

- Fill buckets with your compost feedstocks by packing material into each bucket as tightly as possible:

- Fill 24 buckets with “browns”—shredded leaves, wood chips, shredded cardboard, cut up plant stems (these are examples—your feedstocks may vary)

- Fill 12 buckets with “greens”—shredded hay, chopped up weeds, green leaves, food scraps (these are examples—your feedstocks may vary)

- Fill 4 or more buckets with “high nitrogen”—alfalfa hay or cracked corn or buckets of “clean” manure or fresh beer mash or other cracked grains

- Write down how many buckets you have of each different material you are using, and estimate, if necessary, the percentage brown/green in each bucket. For example: shredded leaves and branches might be 75% green and 25% brown (or ¾ bucket green and ¼ bucket brown)

- WHEN YOU HAVE YOUR BUCKETS FILLED WE WANT TO REVIEW  YOUR RECIPE BEFORE PROCEEDING

- Fill all buckets except the “high nitrogen” at least ¾ full with water; filling until you see water at the bucket edge is fine

- Let materials sit in water for 24 hours

- 2-4 hours before you’re ready to put your pile together, add water to your 4 or more high nitrogen buckets to soak your high N

Putting your pile together:

- Have one geobin or wire round set up on your pallet that is covered with hardware cloth

- Lay out a plastic tarp very close to your geobin or round wire setup, with your buckets nearby. Ideally your tarp is between your two geobin /round wire setups so that when it’s time to turn your pile you are pulling your materials onto your tarp and into the second geobin/round wire setup easily

- Bring one-quarter or one-third of each kind of material to your plastic tarp, dump so that the water runs off, and mix the materials together with your pitchfork (or your hands). If you have different kinds of green and brown materials, use some proportion of all kinds in each one-fourth or one-third you mix. In each one-fourth or one-third also include one-fourth or one-third of your high N. * Even though you’re mixing only part of your compost feedstock at a time, the goal is to have a uniformly mixed overall pile so that all ingredients are evenly distributed

- Add the one-third (or one-fourth) of materials into your geobin or round wire setup via pitchfork

- Repeat above until you have dumped all of the buckets and mixed all of your feedstock materials together and forked them into the geobin/round wire setup

- Insert your compost thermometers into the center of your pile: one thermometer from the top, and two from the sides, making sure the thermometer tips are in the center of the pile

- Cover the full pile with your canvas tarp and secure with bungies

- Write down ambient temperature, temperature registering on your compost thermometers, and date and time

- Clean your plastic tarp and all of your buckets by hosing them off/hosing them out—do this immediately so that materials don’t stick to the buckets, thereby creating possible biofilms

- Wait, expectantly…  :)

Monitoring your pile:

- Check your compost temperatures 6 or 8 hours after you assemble your pile and keep checking every few hours when the temperature begins to rise

You will turn your compost:

72 hours after 131F or

48 hours after 150F or

24 hours after 160F or

12 hours after 165F or

IMMEDIATELY at 170F

- Writing down time and temperature is crucial. Follow the time interval for turning based on the highest temperature your compost reaches

- So: When the temperature hits 131F (average of the 3 thermometers) write down date and time (and text me—photos of thermometers are easiest way to do this)

- Keep monitoring closely once pile reaches 131F and get ready to turn as soon as the time/temperature interval has been reached. Write down ambient temperature, compost thermometer temperature readings, and date and time. Keep a log of some sort, and also take pics and text them

Turning your pile:

- When it’s time to turn, first remove your canvas tarp and lay out your plastic tarp. Since the materials that have been in your hot center are now pathogen and weed seed free, you want to be careful not to contaminate those materials from your pile with debris from the ground around you

- Turning involves mentally dividing the pile into thirds and shifting the thirds in an orderly way, following the same process for each turn so that you are revolving each third through the hot middle of your pile

- Take off the geobin from around your pile and set it aside and lay out the plastic tarp

- Have your second geobin set up on its pallet very near your first pallet/geobin and beside your plastic tarp

- Remove the top third of your compost and place it on the plastic tarp

- Note moisture level: when you squeeze the compost do you get a drop of water? If so, no need to add water at this turn. Very likely you will add water to each third of your compost pile (lightly spraying with a hose is fine) after you’ve placed it into the second compost bin

- Remove the middle third of your compost and place it on the bottom of your second compost bin

- Add the top third of your compost from the plastic tarp into the middle of your second compost bin

- Add the bottom third of your compost onto the top of your second compost bin

- Put your thermometers in place in the middle of your newly constructed compost pile  and cover your compost with canvas tarp

- Write down ambient temperature, compost pile temperatures from center, and date and time

Begin monitoring your pile again:

- After one turn, your compost pile should heat up above 131F within 24 hours. Begin checking temperature 6-8 hours after you have turned your pile

- Monitor your pile as previously, writing down ambient temperature, compost thermometer temperature readings, and date and time. Take pics of the temperature readings and text them

- When your pile has hit time and temperature interval, get ready to turn your pile a second time, following the same process as previously

Turn your pile a second time, same process as previously!

- Again note moisture level: when you squeeze the compost do you get a drop of water? If so, no need to add water at this turn. Very likely you will add water to each third of your compost pile (lightly spraying with a hose is fine) after you’ve placed it back into the original compost bin

Begin monitoring your pile after second turn:

- At this point, your pile needs to hit time and temperature intervals for this second time so that all three thirds of your pile have gone through the hot center of the pile

- Record ambient temperature, compost thermometer temperature readings, and date and time. Take pics of the temperature readings and text them

- Continue to monitor and turn your pile in the same way as previously until the compost pile temperature drops below 131F

- Also monitor your compost pile for moisture and add water via hose as needed

- Continue turning as often as needed, depending on your temperature readings

Finishing your compost:

- When your compost pile drops below 131F—congratulations! Your cooling pile needs to sit until it returns to ambient temperature. To have a significantly fungal pile you will want your pile to sit at ambient temperature for one to three months, depending

- During this time, continue to monitor moisture level. Add water as needed to keep the pile at 50-60% moisture level, as determined by squeeze test.

Review what you learned about feedstocks and process and begin another pile when you’re ready!